# Hints and tips for using the Neoway M590

1. Your software reads from the M590 responses like “OK” , “ERROR” etc.    Instead of parsing these ascii string words, save code and use instead numerical codes like “0” and “4”

ATV=0            switches it on        ATV=1 puts it back to ASCII text

2.        Find out call status info like battery charge level

AT+CIND=?    will ask what’s available

+CIND: ("battchg",(0-5)),("signal",(0-5)),("service",(0,1)),("sounder",(0,1)),("message",(0,1)),("call",(0,1)),("roam",(0,1)),("smsful",(0,1)),("gprs",(0-2)),("callsetup",(0-3)),("callheld",(0-2)),("cellservices",(0-3))

AT+CIND?        willl read status

+CIND: 5,2,1,0,0,0,1,0,2,0,0,1

(battery charge is “5”, signal BER is “2”, service is “1” on, etc etc)

3. Get event-driven reports during a call.      Send AT+CMER: 2,0,0,2,0

4. Dial out star-hash codes like you would using a phone.

ATD*#06#              = IMEI number

5.    Read Operator names

AT+COPN

+COPN: "360110","C&W"

+COPN: "42101","SabaFon"

+COPN: "42102","MTN"

+COPN: "22002","ProMonte"

+COPN: "22003","YUMTS, YUG03" ………

6.    Show the caller’s number who’s calling the M590 SIM number

AT+CLIP=1

7.    Show info about GPRS

AT+CGEREP=1,1          and    AT+CGREG=1

8.      Detach GPRS            Saves power?

AT+CGATT=0