TJ DCR Project -Mixer / Diplexer Boa

| - Gather the components |  |
| --- | --- |
| - Layout the board |  |
| - Write on it! It will help. | d |
| - Solder in the diodes – watch the direction -anode to cathode |  |
| - Transformers – The tap is pre-solderd, primary is twisted, the other tow leads are loose |  |
| - RF transformer – solder the tap to ground |  |
| - One loose end to bottom pad, the other loose end to top pad |  |
| - Untwist the primary winding – solder one end to the RF pad and one end to ground |  |
| - LO Transformer – solder the tap to the AF PAD |  |
| - One loose ending goes to the far pad – you may need to pull the transformer over slightly |  |
| - The other loose winding goes to the near pad |  |
| - Untwist the primary and solder one lead to the pad and one to groun |  |
| - Solder on some test leads. Ready to test |  |