- Add decoupling cap to vbus measurement

- Add heat sinking capability

- Add battery/bus current measurement

- Add third half bridge for 3 Phase Motors

- Lilygo T7 S3 Instead?

- Add TVS Diodes

- Add Reverse polarity protection

- Use IFX007TAUMA1instead

- GPIO25 cannot be used for analog measurements

- Do not use GPIO 12 (strapping pin)

- Add low side switch to turn on and off ext 5V to reboot Bluetooth

- Add External display I2C connections?

- Need to buy:

  - 0603 zero ohm

  - 10k NTC Thermistors

  - 1mOhm shunt

  - 0.1uF 0603 caps

  - Desolder braid

  - Fume extractor

  - Smaller diameter solder

With 3 Automotive Light Bulbs as resistive load:

12.8A no switching FETs and Current Sensor about 65°C (45°C Rise)

58mv across Current Sensor (4.5mOhm)

81mv across Q1 HS (6.32mOhm)

89mv across Q2 LS (6.95mOhm)

34mv between TP4 and TP6 (B+ inputs of Q1 and Q2)

1.7mV between TP3 and TP5 (B- inputs of Q1 and Q2)

200mV from B+ input to MTR2 (2.568W) (Q1 HS Conducting is worst case) (15.6mOhm total) (9.3mOhm PCB)

95mV from B- input MTR1 (1.22W) (7.4mOhm)

Polarity Swapped:

105mV from B+ to MTR1 (1.3W)

145mV from B- to MTR2 (1.8W)

97.6% Efficient at 12.8A

Vbus=10V,12.28A switching Q2 at 50kHz, current entering Q2, LS=90%, HS=10%, Q2=98°C, Q1=70°C, Rise Time=350ns, Fall time= 300ns, Current Sensor=73°C

With inductive motor load at 15A voltage overshoot 28%, at Vbus=10V

With Vbus=24V, 15A Current entering Q2 LS=15% duty, 28V peak overshoot, 154ns fall time, 282ns rise time, Q2 125°C

The thermistor reads 90°C while one Half bridge is 80 and the other is 125°C

September 6th 2024 – Changed Q2 slew rate resistor from 510R to 1.1R and added back 220nF for C5 and C6

At 28V Vbus, 15A current entering Q2, the hard switching off of the LS took 250ns (rise time) and the overshoot was 28.2V to 31.6V (12%)

September 7th 2024 – Added 2mOhm INA260 bypass split to increase overall current handling capability

At 42A, 10.7V bus, overshoot up to 13.6V (27%), hard switching off of the LS took 250ns (rise time)

With a fan blowing on backside, 10kHz, Vbus=10.7V

42A, 17secs

32A, 64secs

28A, 5mins (300secs)

September 20th 2024 – PCBA #2 – Only 3A DC PTC

Vbus=18V, SF=20kHz, 12A Phase, 91°C thermistor

Vbus=18V, SF=10kHz, 14.2A Phase, 92.4°C thermistor

Vbus=18V, SF=5kHz, 14.2A Phase, 83°C thermistor