Scout v2.2

Project Guide: What You Can Build Out of the Box

Zero additional hardware required. Open the box. Plug in USB. Build.

# What’s On Board

Every project below uses only the sensors, storage, and output hardware built into Scout. No breakout boards. No breadboard. No jumper wires. No extra purchases.

| Component | Part | What It Measures |
| --- | --- | --- |
| Environment Sensor | BME280 | Temperature, humidity, barometric pressure |
| Accelerometer | LIS3DH | 3-axis acceleration, tilt, vibration, freefall |
| Light Sensor | VEML7700 | Ambient light level (lux) |
| Real-Time Clock | DS3231 | Date/time with battery backup |
| Data Storage | Micro SD slot | Log data to CSV files |
| Power Output | 30V/2A PWM | Control pumps, fans, solenoids, LED strips |
| Processor | RP2350B | Dual-core 150MHz, 520KB SRAM |

# Beginner Projects

First day with a microcontroller. No experience required.

1. Hello, World Weather Station

Read your room’s weather and print it to the serial monitor

Sensors used: BME280

Difficulty: Absolute beginner — 10 minutes

The classic first project. Read temperature, humidity, and barometric pressure and display the values over USB serial. Five lines of code. Takes about ten minutes from unboxing to seeing real data. This is the project that every other board makes you buy $20 in breakouts for.

2. Light Level Monitor

Measure ambient brightness and trigger actions at thresholds

Sensors used: VEML7700

Difficulty: Absolute beginner — 15 minutes

Read lux values from the onboard light sensor. Print them to serial. Set a threshold: if the room gets dark, light up the onboard LED. First introduction to conditional logic with real sensor data.

3. Digital Spirit Level

Use the accelerometer to measure tilt on two axes

Sensors used: LIS3DH

Difficulty: Beginner — 20 minutes

Read X and Y acceleration values, convert to tilt angle, display over serial. Hold Scout flat, then tilt it and watch the numbers change. Great introduction to accelerometers and basic trigonometry. Print arrows showing which direction to tilt to level.

4. Timestamped Data Logger

Log sensor data to the SD card with real date and time stamps

Sensors used: BME280, DS3231, SD card

Difficulty: Beginner — 30 minutes

Read temperature and humidity every 30 seconds. Get the real time from the RTC. Write a CSV line to the SD card: timestamp, temperature, humidity, pressure. Pop the SD card into your laptop, open in Excel. You now have a professional data logger. This project alone would cost $50+ in breakouts on any other board.

5. Desk Environment Dashboard

Read all four sensors at once and display a formatted status screen

Sensors used: BME280, LIS3DH, VEML7700, DS3231

Difficulty: Beginner — 30 minutes

Poll every onboard sensor. Format a clean dashboard output over serial: time, temperature, humidity, pressure, light level, and board orientation. Refresh every second. First experience reading multiple I2C devices on a shared bus. The fact that this works without wiring a single connection is the whole point of Scout.

# Intermediate Projects

You know the basics. Time to build systems that do real things.

6. Weather Station with SD Logging

Continuous environmental recording with hourly statistics

Sensors used: BME280, VEML7700, DS3231, SD card

Difficulty: Intermediate — 1 hour

Log temperature, humidity, pressure, and light level every minute to the SD card. Every hour, calculate min/max/average and write a summary row. Detect rain probability by watching barometric pressure trends. Run it for a week, pull the card, and graph a real weather dataset. Add the light sensor data to track cloud cover.

7. Automatic Fan Controller

Turn on a fan when temperature exceeds a threshold

Sensors used: BME280, PWM output

Difficulty: Intermediate — 1 hour

Read temperature. When it crosses your set point, ramp up a 12V fan through the PWM output. Use proportional control: hotter = faster fan. This is a real closed-loop control system built with zero external components. The sense-act loop that no other $28 board can do.

8. Plant Watering Pump Controller

Scheduled watering with environment-aware adjustments

Sensors used: DS3231, BME280, PWM output

Difficulty: Intermediate — 1-2 hours

Use the RTC to trigger a 12V water pump at set times (e.g., 7 AM and 6 PM). Log each watering event to the SD card. Adjust duration based on temperature—hotter days get more water. This is home automation with zero cloud, zero WiFi, zero subscriptions. Just a board, a pump, and tubing.

9. Motion and Vibration Alarm

Detect movement or vibration and trigger an alert

Sensors used: LIS3DH, DS3231, SD card

Difficulty: Intermediate — 1 hour

Configure the LIS3DH interrupt to fire on motion above a threshold. Log every event with a timestamp. Use the PWM output to drive a piezo buzzer or an LED strip as an alarm. Build a simple security logger that records exactly when and how hard something was disturbed.

10. Sunrise/Sunset Light Controller

Automatically control lights based on ambient brightness

Sensors used: VEML7700, DS3231, PWM output

Difficulty: Intermediate — 1-2 hours

Read ambient light levels. When lux drops below your threshold at dusk, smoothly ramp up a 12V LED strip through the PWM output. At dawn, ramp it back down. Log the on/off times to SD with real timestamps. A complete outdoor lighting automation system.

11. Earthquake/Tremor Logger

Continuously log acceleration data and detect seismic events

Sensors used: LIS3DH, DS3231, SD card

Difficulty: Intermediate — 1-2 hours

Sample the accelerometer at high rate. Establish a noise baseline, then flag any event that exceeds it. Log timestamped magnitude data to the SD card. Set up the board in your basement and leave it running. Actual seismology with actual data on a $28 board.

12. Fridge/Freezer Temperature Guardian

Monitor cold storage and alert on temperature excursions

Sensors used: BME280, DS3231, SD card, PWM output

Difficulty: Intermediate — 1 hour

Place Scout in a cold environment. Log temperature continuously. If the temperature rises above a safe threshold (power outage, door left open), trigger an alert through the PWM output—buzzer, light, whatever you wire to the screw terminal. The SD card log gives you a complete cold chain record with timestamps. Useful for anyone storing medication, food, or lab samples.

# Advanced Projects

Multi-sensor systems, data science, and real automation.

13. Indoor Air Quality Index Calculator

Compute comfort indices from combined sensor data

Sensors used: BME280, VEML7700

Difficulty: Advanced — 2-3 hours

Combine temperature and humidity into a heat index. Calculate dew point. Estimate comfort zone using ASHRAE standards. Display a real-time IAQ score. Log trends over days. Add light level data to assess workspace quality. Publish your algorithm on GitHub for others to use.

14. Altimeter and Vertical Speed Indicator

Calculate altitude from barometric pressure and detect elevation changes

Sensors used: BME280, LIS3DH

Difficulty: Advanced — 2-3 hours

Use the barometric pressure from the BME280 to calculate altitude above sea level. Differentiate for vertical speed (rate of climb/descent). Cross-reference with the accelerometer to filter noise. Mount it on a drone, in an elevator, or carry it on a hike. Log the altitude profile to SD.

15. Greenhouse Automation System

Complete growing environment controller with logging

Sensors used: BME280, VEML7700, DS3231, SD card, PWM output

Difficulty: Advanced — 3-4 hours

Monitor temperature, humidity, and light levels continuously. Control a ventilation fan on temperature. Control supplemental grow lights on lux levels. Log everything with timestamps. Calculate growing degree days. Generate daily summary statistics. This is a complete agricultural monitoring and control system on one board for $28.

16. Step Counter and Activity Tracker

Detect walking, running, and idle states from acceleration patterns

Sensors used: LIS3DH, DS3231, SD card

Difficulty: Advanced — 3-4 hours

Apply a low-pass filter to the accelerometer Z-axis to detect the periodic signal of footsteps. Count steps, estimate distance using stride length calibration. Classify activity states (idle, walking, running) by acceleration magnitude. Log a daily activity profile. Introduction to signal processing on real data.

17. Weather Prediction Engine

Predict weather trends from barometric pressure and humidity patterns

Sensors used: BME280, DS3231, SD card

Difficulty: Advanced — 4-5 hours

Sample pressure every 10 minutes. Calculate the 3-hour pressure trend. Apply Zambretti’s forecasting algorithm (a real meteorological method from the 1920s) to predict weather from the rate and direction of pressure change combined with current conditions. Log predictions alongside actual observations and score accuracy over time.

18. PID Temperature Controller

Implement a real proportional-integral-derivative control loop

Sensors used: BME280, PWM output

Difficulty: Advanced — 3-4 hours

Build a PID controller that maintains a target temperature by modulating a heating element or fan through the PWM output. Tune Kp, Ki, and Kd gains. Log setpoint, process variable, and control output over time. Plot the response curve. This is actual control systems engineering on a $28 board. Engineering students pay thousands in lab fees to do this exact exercise.

19. Multi-Zone Environmental Survey

Move Scout between locations and build a spatial environment map

Sensors used: BME280, VEML7700, LIS3DH, DS3231, SD card

Difficulty: Advanced — 2-3 hours

Carry Scout through a building, pausing at each room to record conditions. Use the accelerometer to detect when you’ve stopped moving. Log environment data with location tags. Build a heatmap of temperature, humidity, and light levels across a floor plan. Facilities managers pay thousands for commercial tools that do this.

20. Resin Printer Environment Monitor

Track and control conditions in your 3D printing enclosure

Sensors used: BME280, VEML7700, DS3231, SD card, PWM output

Difficulty: Advanced — 2-3 hours

Resin 3D printing is sensitive to temperature and UV exposure. Monitor enclosure temperature and humidity continuously. If temperature drops below the resin’s working range, trigger a heater through the PWM output. Log conditions through every print job. Correlate environmental data with print quality. Use the light sensor to detect when the UV cover is removed.

# Classroom and STEM Projects

Designed for teachers, makerspaces, and after-school programs.

21. Science Fair Weather Station

Week-long data collection with charts and analysis

Sensors used: BME280, VEML7700, DS3231, SD card

Difficulty: Beginner — 30 min setup, 1 week collection, 1 hour analysis

Set up Scout to log weather data for 7 days. Pull the SD card, import the CSV into Google Sheets or Excel. Build charts showing daily temperature curves, humidity patterns, and barometric pressure trends. Correlate light levels with cloud cover. A complete science fair project that generates real, original data. No internet required.

22. Physics: Acceleration and Free Fall

Measure gravitational acceleration and demonstrate Newton’s laws

Sensors used: LIS3DH, DS3231, SD card

Difficulty: Beginner to intermediate — 1 class period

Hold Scout still and measure 1g. Tilt it and decompose gravity into X/Y/Z components. Drop it (gently, onto a pillow) and capture the freefall signature where all axes read near zero. Log high-rate acceleration data and calculate g from the measurements. Real physics with real numbers.

23. Math: Exponential Decay in Real Time

Watch Newton’s Law of Cooling happen live

Sensors used: BME280, DS3231, SD card

Difficulty: Intermediate — 1 class period

Heat Scout gently (hand warmth is enough). Log temperature as it cools back to ambient. Plot the curve. Fit an exponential decay function. Calculate the time constant. This is calculus made tangible. Students can see the math happening in real data from a sensor they’re holding.

24. Engineering: Closed-Loop Control Lab

Build and tune a real control system

Sensors used: BME280, PWM output, SD card

Difficulty: Intermediate to advanced — 2 class periods

Connect a small fan to the PWM output. Set a target temperature. Implement on/off control first (bang-bang), observe the oscillation. Then implement proportional control and watch it smooth out. Log both experiments and compare the response curves. This replaces a $500 control systems lab kit.

25. Data Science: Sensor Fusion and Correlation

Explore statistical relationships between environmental variables

Sensors used: BME280, VEML7700, LIS3DH, DS3231, SD card

Difficulty: Advanced — multi-day project

Collect a week of multi-sensor data. Import into Python or Excel. Calculate correlation coefficients between temperature and humidity, light and temperature, pressure and time of day. Build scatter plots. Introduce students to the concept that sensors can contradict each other and data requires interpretation.

# The Count

25 complete projects from a single $28 board with zero additional hardware purchases. The same project list on an Arduino Uno R4 WiFi would require $50-80 in breakout boards, a breadboard, and 20+ jumper wires.

Every project above generates real data from real sensors, controls real loads, and logs to real storage. No simulations. No serial-monitor-only demos. Scout was designed to close the gap between reading a sensor and doing something useful with the data.

This list uses only onboard hardware. Add a Qwiic sensor, a servo, or a display through the expansion headers and the project count is effectively unlimited.

Open Source is Love