IV-25 VFD Clock

Font Code Transfer — From the Content Session to the Firmware Session

# Overview

The content session is transferring all font-related code to the firmware session as requested. This document describes what is being transferred, the working rendering functions developed and tested by the content session, and the interface the content session would like back.

Cliff will copy the font files to the firmware session's sketch folder.

# Files Being Transferred

Two files transfer wholesale:

- font.h — declares extern const uint8_t asciiFont[95][7]

- font.cpp — defines the full 95-character ASCII bitmap array, covering all printable ASCII characters 32 (space) through 126 (~)

The bitmap format in font.cpp:

- Each character is 4 pixels wide, 7 pixels tall

- Indexed as asciiFont[charCode - 32][row]

- Row 0 = top pixel, row 6 = bottom pixel

- Within each row byte: bit 3 = leftmost column, bit 0 = rightmost column

The digit font (digitFont) in animation.ino is a separate 10-character font used by the clock face renderer. It is not the same as asciiFont and is not part of this transfer.

# Rendering Functions — Tested and Working

These two functions were developed and tested by the content session. They produce correct pixel output on the real IV-25 hardware. The firmware session should use this code directly rather than rewriting it — the pixel geometry has been carefully verified.

## drawChar()

// Draw a single ASCII character at startCol.

// Characters are 4 pixels wide + 1 pixel gap = 5 pixels per slot.

// Out-of-bounds pixels are silently clipped by setPixel().

void drawChar(char c, int startCol) {

if (c < 32 || c > 126) return;

int idx = (int)c - 32;

for (int row = 0; row < 7; row++) {

uint8_t rowData = asciiFont[idx][row];

for (int col = 0; col < 4; col++) {

if ((rowData >> (3 - col)) & 1)

setPixel(startCol + col, row, true);

}

}

}

## drawString()

// Draw a null-terminated ASCII string starting at startCol.

// Returns the column just past the last character drawn.

int drawString(const char* str, int startCol) {

int col = startCol;

while (*str) {

drawChar(*str, col);

col += 5;  // 4 pixels wide + 1 gap

str++;

}

return col;

}

# Interface Required by the Content Session

After the transfer, the content session requires the following functions to be available as callable interface functions:

// Draw single ASCII character at startCol.

void drawChar(char c, int startCol);

// Draw null-terminated ASCII string starting at startCol.

// Returns column just past last character drawn.

int drawString(const char* str, int startCol);

// Return pixel width of a null-terminated string.

// Implementation: return (int)strlen(str) * 5;

// Needed for centering calculations.

int getStringWidth(const char* str);

getStringWidth() is new — not currently in the content session's code but needed for layout calculations. The implementation is trivial: return strlen(str) * 5.

# One Interface Constraint

drawChar() must remain available as a directly callable function — not just used internally by scrollText(). The content session has rendering needs beyond full-string scrolling that require placing individual characters at specific positions.