Organization of the Specification

General design

This project is for a clock consisting of an ESP32 programmed in Arduino C++ and a display consisting of a number of 7 dot VFD tubes mounted vertically.  The program will translate text, like the time, into a set of dots to turn on or off.

To help with the challenges of making a very wide display with lots of tubes, the hardware is divided into PC boards that have 16 of these 7 dot VFD tubes (which are IV-25 from the 1980’s).  One of the boards will be a “master” that will run the primary clock application and that will control some number of “slave” boards.  Each board has an board number.  Board zero is the master board.  Boards 1 through 7 are the slave boards.  There may be a maximum of 8 boards total, though the system will operate with any number of boards from 1 to 8.  The master board will communicate with the slave boards through an RS485 bus, under control by the master, in order to tell the slave boards what to display on their tubes.

In order for the clock software to maintain accurate time, it will use both a DS3231 real-time-clock module as well as NTP.  The clock will use a VL53L1x module to sense whether anyone is in front of the clock, and turn off the clock if not for a while.  The clock will use a TSL2591 module to detect the level of ambient lighting and dim the tubes as appropriate.

Due to the wide variance in brightness of the IV-25 tubes (they are 40+ years old and were manufactured in the Soviet Union, so their quality is quite variable), the clock will use a PWM approach on an individual dot basis.  The clock needs to do this PWM continuously, and a user friendly UI will allow the user to calibrate the brightness of the boards, tubes, and dots to get as even a brightness as possible.

For now, the information displayed on the tubes will be just the current time.  When everything is working, we will consider displaying other information and doing visual tricks like scrolling.  The code for displaying text on the tubes will be in a separate file, with the interfaces designed so that this code will not need top understand the workings of the clock.

Specification files

The specifications for this project are broken into the following files:

01 General:

This particular document is the “General” file.  It describes the purpose of this project and the organization of the specification files.

02 Approach:

This file describes the approach I would like to take to design and implement this clock project.

03 Hardware:

This file has the specifications for the hardware in this clock project.

04 Software:

This file has the specifications for all the general aspects of the clock application.

05 Clock:

This file describes how time management, resilience, and accuracy are accomplished.

06 RS485:

This file describes how the RS485 bus is used, what information is communicated, what is the protocol for such communication, and the subroutines and other things necessary to make it work.

07 TubePWM:

This file explains how PWM is accomplished on a per dot basis.  It also contains the specifications for the UI necessary to calibrate the PWM settings.

08 I2C:

This file has the specifications for all the I2C devices in this clock.

09 Display:

This file describes the how text is rendered on the VFD tubes.

10 UI:

This file describes the UI for the clock program.

These specification files are meant to be read in order, starting with “01 General”, followed by “02 Approach” and so on.  The two digits at the beginning of the file name indicate this ordering.

For each of these specification files, I will keep multiple versions by appending a version number to the file name.  So for example, the file “05 Clock 2” is the “clock” specifications, version 2.