# Clock Time Management

This system will depend on and display the contents of the ESP32's time of day (TOD) clock. However, there will be code running in the ClockTime thread that will periodically adjust the ESP32's clock so that it is as accurate as possible.

There are two sources of more accurate time: A DS3231 RTC module on the I2C bus, and NTP. A challenge is that one or the other may not be available, so this design has to be resilient (one has to be available for this system to show accurate time, but if neither is available the clock should operate with whatever is in the TOD clock).

The proposed approach is as follows:

At startup the TOD clock will initialize at zero time (whatever that is – I don't care).

The ClockTime thread will monitor the TOD clock for DST transitions and adjust the TOD clock as appropriate when transitions occur. For now, DST rules are hardcoded for US Eastern time zone (second Sunday of March forward, first Sunday of November back).

The time that is stored in the DS3231 and the time that comes from NTP is in Universal Time (UTC).

Every hour, ClockTime will read the UTC time from the DS3231 RTC, convert to the proper local time (including DST) and put that into the TOD clock.

Once a day, ClockTime will fetch the current UTC time from the internet with NTP. This UTC time will be written both into the DS3231 RTC and into the TOD clock (with conversion to local time and considering DST).

For now, WiFi credentials for NTP access are hardcoded: SSID="B" and password="70317031".

This entire clock management runs on the master board only. The ClockTime module will read the TOD clock and cause it to be displayed on the tubes.

# NTP Time Synchronization

NTP time synchronization uses configTime.  Use waitForNtp(10000) to make sure that the NTP synchronization call has successfully completed.  We had significant difficulty with getting proper NTP time because of collisions with ???

The system tries multiple NTP servers in sequence (time.nist.gov, time.google.com, time.cloudflare.com, pool.ntp.org) with 3-second timeout per server. UTC time is written to DS3231, then converted to local time (UTC + timezone offset + DST if applicable) for the system clock and display.

NTP sync occurs 5 seconds after boot and then daily. If NTP fails, the clock continues using DS3231 time. Users can manually set time via the SET_TIME console command.