# Management of the I2C Bus

The I2C bus is not protected from read or write collisions within a single board. So all access to I2C devices should be protected with a single mutex for the entire I2C bus. This mutex must be acquired before any device read or write and released after the transaction completes.

There is an I2C address collision between the VL53L1x and TSL2591 devices. The solution to this problem will be extracted from the test program provided separately.  The solution involves changing the address of the VL53L1x at startup as the very first command before the TSL2591 has a chance to get confused with the VL53L1x.