# Display Format and Font Specification

This document specifies how the time is rendered on the VFD tube display, including the font design and layout.

## Display Layout

The initial system consists of 32 tubes (2 boards: master + 1 slave). The time will be displayed in HHMMSS format with no colons between the digit pairs.

Layout breakdown:

6 digits (HH MM SS) × 4 pixels per digit = 24 pixels

5 inter-digit spaces × 1 pixel = 5 pixels

2 extra blank tubes (one between HH and MM, one between MM and SS) = 2 pixels

Total: 24 + 5 + 2 = 31 pixels used, with 1 pixel unused

Example rendering for time 12:34:56 would appear as: "1 2  3 4  5 6" where single spaces are between digits and double spaces separate digit pairs.

## Font Specification

Font type: 4×7 pixel bitmap font (4 pixels wide, 7 pixels tall)

# Display Font

The clock uses a 4×7 pixel font for digits 0-9. Each character occupies 4 columns (bits 0-3 used for pixels, bit 4 always 0 for spacing between digits). The font is stored in font5x7[10][7] array (variable name retained for compatibility). Row 0 is top, row 6 is bottom. Bit 0 is leftmost pixel, bit 3 is rightmost pixel. The font includes one blank column (5th column) between adjacent digits, providing clean spacing. Total layout: 1 blank + digit(4) + blank(1) + digit(4) + ... = 30 columns for 6 digits.

Character set: '0' through '9' (digits only for initial implementation)

Storage format: Each character stored as an array of 7 bytes, where each byte represents one horizontal row of the character. For a 4-pixel wide character, only the lower 4 bits of each byte are used. Bit 0 = leftmost pixel in that row.

Memory allocation: Font data should be stored in normal RAM (not PROGMEM). The ESP32 has sufficient RAM that optimization for program memory is unnecessary.

Note: The font should be designed during code generation to be readable and well-proportioned within the 4×7 constraint.

## ComputeDisplay Rendering

ComputeDisplay reads the current TOD clock value and extracts the hours, minutes, and seconds. Each digit is looked up in the font table and the resulting 4×7 pixel pattern is written to allDisplayBits at the appropriate position.

The physical-to-logical coordinate mapping described in the Hardware specification must be applied so that the leftmost digit (the tens digit of hours) appears at the physical leftmost edge of the display.

The layout uses 1 pixel spacing between adjacent digits and 2 pixels (1 inter-digit space + 1 extra blank) between digit pairs (HH|MM and MM|SS).

# Ambient Brightness Control Specification

## Overview

The ambient brightness control system automatically adjusts display brightness based on ambient light levels detected by the TSL2591 sensor. It uses a linear interpolation formula with user-configurable parameters stored in preferences.

## Brightness Algorithm

Parameters (stored in preferences):

• brightnessMinReading (default 25): Sensor reading in truly dark room

• brightnessMaxReading (default 100): Sensor reading for normal brightness

• brightnessMinPercent (default 80): Display brightness % in dark room

• brightnessMaxPercent (default 100): Display brightness % in normal/bright light

Linear Brightness Formula:

1. If reading >= brightnessMaxReading: brightness = brightnessMaxPercent (typically 100%)

2. If reading <= brightnessMinReading: brightness = brightnessMinPercent (typically 80%)

3. If between min and max: brightness = brightnessMinPercent + (reading - minReading) × (maxPercent - minPercent) / (maxReading - minReading)

Example Calculation:

• Dark room (reading=25): brightness = 80%
• Mid-range (reading=62): brightness = 90%
• Normal light (reading=100+): brightness = 100%

## PWM Duty Cycle Conversion

The BLANK pin on MAX6921 controls display blanking. HIGH = display OFF, LOW = display ON. To achieve the desired brightness percentage, the PWM duty cycle is inverted:

dutyCycle = 100 - brightnessPercent

pwmValue = (dutyCycle × 255) / 100

Examples:

• 100% brightness: dutyCycle=0%, pwmValue=0 (BLANK always LOW, display always ON)
• 80% brightness: dutyCycle=20%, pwmValue=51 (BLANK HIGH 20% of time)
• 50% brightness: dutyCycle=50%, pwmValue=127

## Preferences Storage

Namespace: "brightness"

Stored Parameters:

• minRead (int): brightnessMinReading
• maxRead (int): brightnessMaxReading
• minPct (int): brightnessMinPercent
• maxPct (int): brightnessMaxPercent

Functions:

• loadBrightnessParams(): Called during master initialization, loads parameters from preferences

• saveBrightnessParams(): Called when user changes parameters via console, saves to preferences

## Brightness Preferences Storage

Namespace: "brightness"

Purpose: Store user-configurable parameters for ambient brightness control algorithm

Stored Keys:

• minRead (int, default 25): Sensor reading for dark room
• maxRead (int, default 100): Sensor reading for normal brightness
• minPct (int, default 80): Display brightness % in dark room
• maxPct (int, default 100): Display brightness % in normal light

Functions:

• loadBrightnessParams(): Called in setup(), loads parameters or uses defaults
• saveBrightnessParams(): Called when user changes parameters via console commands

Usage Pattern:

User adjusts parameters via console commands (BRIGHTNESS_SET_MIN_READ, etc.), saveBrightnessParams() immediately writes to NVS, parameters persist across power cycles and are loaded at next boot.

## Console Commands

BRIGHTNESS_MONITOR

Displays live sensor readings every 500ms with calculated brightness percentage. Shows which zone the reading falls into (below min, between min/max, or above max). Press any key to stop monitoring.

BRIGHTNESS_PARAMS

Displays current brightness parameters and lists available SET commands.

BRIGHTNESS_SET_MIN_READ <value>

Set minimum sensor reading threshold (dark room). Range: 0-65535. Saves to preferences.

BRIGHTNESS_SET_MAX_READ <value>

Set maximum sensor reading threshold (normal brightness). Range: 0-65535. Saves to preferences.

BRIGHTNESS_SET_MIN_PCT <value>

Set display brightness percentage in dark room. Range: 0-100. Saves to preferences.

BRIGHTNESS_SET_MAX_PCT <value>

Set display brightness percentage in normal/bright light. Range: 0-100. Saves to preferences.

## Thread Implementation

Thread: threadAmbientBrightness (master only)

Priority: 1 (same as ClockTime and ComputeDisplay)

Algorithm Steps:

1. Read TSL2591 sensor (with i2cMutex)

2. Calculate brightness percentage using linear formula

3. Convert to PWM duty cycle: duty = 100 - brightness

4. Convert to 8-bit value: pwm = (duty × 255) / 100

5. Format broadcast command: sprintf(cmd, "$T%02X", pwmValue)

6. Call RS485WriteCommandBroadcast(cmd) to send to all boards

7. Sleep 500ms, repeat

## RS485_BRIGHTNESS Command

Command: $Txx\0

Purpose: Set display brightness (BLANK pin PWM duty cycle) on all boards simultaneously

Direction: Master → All slaves (broadcast) + Master → Self (via ring buffer)

Format: $Txx\0 where xx is two-character hexadecimal value (00 to FF)

Board Number: NONE - This is a broadcast command with no board number field

Broadcast Behavior:

The AmbientBrightness thread (master only) sends $Txx\0 ONCE to the RS485 bus via RS485WriteCommandBroadcast(), and all slave boards receive and execute it simultaneously. The master board receives the same command via the command ring buffer.

Processing (All Boards):

When ReceiveCommand receives $Txx\0: (1) Parse hex characters xx to integer 0-255, (2) Call ledcWrite(BLANK_PWM_CHANNEL, value) to set BLANK pin (D8) PWM duty cycle

## RS485 Transmit Mutex (rs485TxMutex)

Multiple threads write to Serial1 (RS485 bus): SendDisplay sends $D commands, AmbientBrightness sends $T commands. The rs485TxMutex synchronizes access to prevent message corruption.

Mutex Creation: rs485TxMutex = xSemaphoreCreateMutex() in initHardware()

Usage Pattern - Batch Transmissions (SendDisplay): Grab mutex once, transmit all changed tubes in batch, release mutex. This keeps related messages together and minimizes mutex overhead.

Usage Pattern - Single Transmission (AmbientBrightness): Uses RS485WriteCommandBroadcast() which handles mutex internally.

## PWM Channel Configuration

Channel 0: Filament driver (20KHz on D4)

Channel 1: BLANK pin brightness control (10KHz on D8)

Configuration:

ledcSetup(BLANK_PWM_CHANNEL, BLANK_PWM_FREQ, BLANK_PWM_RESOLUTION);
ledcAttachPin(BLANK_PIN, BLANK_PWM_CHANNEL);
ledcWrite(BLANK_PWM_CHANNEL, 127);  // Start at 50% brightness

# Ambient Brightness Control Specification

## Overview

The ambient brightness control system automatically adjusts display brightness based on ambient light levels detected by the TSL2591 sensor. It uses a linear interpolation formula with user-configurable parameters stored in preferences.

## Brightness Algorithm

Parameters (stored in preferences):

• brightnessMinReading (default 25): Sensor reading in truly dark room

• brightnessMaxReading (default 100): Sensor reading for normal brightness

• brightnessMinPercent (default 80): Display brightness % in dark room

• brightnessMaxPercent (default 100): Display brightness % in normal/bright light

Linear Brightness Formula:

1. If reading >= brightnessMaxReading: brightness = brightnessMaxPercent (typically 100%)

2. If reading <= brightnessMinReading: brightness = brightnessMinPercent (typically 80%)

3. If between min and max: brightness = brightnessMinPercent + (reading - minReading) × (maxPercent - minPercent) / (maxReading - minReading)

Example Calculation:

• Dark room (reading=25): brightness = 80%
• Mid-range (reading=62): brightness = 90%
• Normal light (reading=100+): brightness = 100%

## PWM Duty Cycle Conversion

The BLANK pin on MAX6921 controls display blanking. HIGH = display OFF, LOW = display ON. To achieve the desired brightness percentage, the PWM duty cycle is inverted:

dutyCycle = 100 - brightnessPercent

pwmValue = (dutyCycle × 255) / 100

Examples:

• 100% brightness: dutyCycle=0%, pwmValue=0 (BLANK always LOW, display always ON)
• 80% brightness: dutyCycle=20%, pwmValue=51 (BLANK HIGH 20% of time)
• 50% brightness: dutyCycle=50%, pwmValue=127

## Preferences Storage

Namespace: "brightness"

Stored Parameters:

• minRead (int): brightnessMinReading
• maxRead (int): brightnessMaxReading
• minPct (int): brightnessMinPercent
• maxPct (int): brightnessMaxPercent

Functions:

• loadBrightnessParams(): Called during master initialization, loads parameters from preferences

• saveBrightnessParams(): Called when user changes parameters via console, saves to preferences

## Console Commands

BRIGHTNESS_MONITOR

Displays live sensor readings every 500ms with calculated brightness percentage. Shows which zone the reading falls into (below min, between min/max, or above max). Press any key to stop monitoring.

BRIGHTNESS_PARAMS

Displays current brightness parameters and lists available SET commands.

BRIGHTNESS_SET_MIN_READ <value>

Set minimum sensor reading threshold (dark room). Range: 0-65535. Saves to preferences.

BRIGHTNESS_SET_MAX_READ <value>

Set maximum sensor reading threshold (normal brightness). Range: 0-65535. Saves to preferences.

BRIGHTNESS_SET_MIN_PCT <value>

Set display brightness percentage in dark room. Range: 0-100. Saves to preferences.

BRIGHTNESS_SET_MAX_PCT <value>

Set display brightness percentage in normal/bright light. Range: 0-100. Saves to preferences.

## Thread Implementation

Thread: threadAmbientBrightness (master only)

Priority: 1 (same as ClockTime and ComputeDisplay)

Algorithm Steps:

1. Read TSL2591 sensor (with i2cMutex)

2. Calculate brightness percentage using linear formula

3. Convert to PWM duty cycle: duty = 100 - brightness

4. Convert to 8-bit value: pwm = (duty × 255) / 100

5. Format broadcast command: sprintf(cmd, "$T%02X", pwmValue)

6. Call RS485WriteCommandBroadcast(cmd) to send to all boards

7. Sleep 500ms, repeat

## RS485_BRIGHTNESS Command

Command: $Txx\0

Purpose: Set display brightness (BLANK pin PWM duty cycle) on all boards simultaneously

Direction: Master → All slaves (broadcast) + Master → Self (via ring buffer)

Format: $Txx\0 where xx is two-character hexadecimal value (00 to FF)

Board Number: NONE - This is a broadcast command with no board number field

Broadcast Behavior:

The AmbientBrightness thread (master only) sends $Txx\0 ONCE to the RS485 bus via RS485WriteCommandBroadcast(), and all slave boards receive and execute it simultaneously. The master board receives the same command via the command ring buffer.

Processing (All Boards):

When ReceiveCommand receives $Txx\0: (1) Parse hex characters xx to integer 0-255, (2) Call ledcWrite(BLANK_PWM_CHANNEL, value) to set BLANK pin (D8) PWM duty cycle

## RS485 Transmit Mutex (rs485TxMutex)

Multiple threads write to Serial1 (RS485 bus): SendDisplay sends $D commands, AmbientBrightness sends $T commands. The rs485TxMutex synchronizes access to prevent message corruption.

Mutex Creation: rs485TxMutex = xSemaphoreCreateMutex() in initHardware()

Usage Pattern - Batch Transmissions (SendDisplay): Grab mutex once, transmit all changed tubes in batch, release mutex. This keeps related messages together and minimizes mutex overhead.

Usage Pattern - Single Transmission (AmbientBrightness): Uses RS485WriteCommandBroadcast() which handles mutex internally.

## PWM Channel Configuration

Channel 0: Filament driver (20KHz on D4)

Channel 1: BLANK pin brightness control (10KHz on D8)

Configuration:

ledcSetup(BLANK_PWM_CHANNEL, BLANK_PWM_FREQ, BLANK_PWM_RESOLUTION);
ledcAttachPin(BLANK_PIN, BLANK_PWM_CHANNEL);
ledcWrite(BLANK_PWM_CHANNEL, 127);  // Start at 50% brightness