# Console Commands (Implemented)

The following console commands are implemented and functional:

• HELP - Display list of available commands

• INFO - Display system information (board role, number of boards, boot time, current time, free heap)

• CALIBRATE - Enter PWM calibration mode (per TubePWM_2.docx specification)

• PERFTEST - Performance test display code (1000 iterations of shiftOutBits and updateDisplay)

• SETDELAYS - Interactively set MAX6921 timing delays for experimentation

• SET_TIME - Manually set current time (format: YYYY MM DD HH MM SS in local time)

The following console commands are not yet implemented:

• SET_TIMEZONE - Set timezone offset

• SET_DST - Enable/disable DST

# Display Font

The clock uses a 4×7 pixel font for digits 0-9. Each character occupies 4 columns (bits 0-3 used for pixels, bit 4 always 0 for spacing between digits). The font is stored in font5x7[10][7] array (variable name retained for compatibility). Row 0 is top, row 6 is bottom. Bit 0 is leftmost pixel, bit 3 is rightmost pixel. The font includes one blank column (5th column) between adjacent digits, providing clean spacing. Total layout: 1 blank + digit(4) + blank(1) + digit(4) + ... = 30 columns for 6 digits.

## Brightness Console Commands

BRIGHTNESS_MONITOR

Purpose: Display live sensor readings and calculated brightness
Format: BRIGHTNESS_MONITOR
Behavior: Shows sensor reading every 500ms with zone indication (dark/interpolated/bright). Press any key to stop.
Example Output: 'Sensor reading: 70 [<=25: 80% brightness]'

BRIGHTNESS_PARAMS

Purpose: Display current brightness parameters and available SET commands
Format: BRIGHTNESS_PARAMS
Displays:
  • Min sensor reading (dark room)
  • Max sensor reading (normal light)
  • Min brightness percent (dark)
  • Max brightness percent (bright)

BRIGHTNESS_SET_MIN_READ <value>

Purpose: Set sensor reading threshold for dark room
Format: BRIGHTNESS_SET_MIN_READ <value>
Range: 0-65535
Example: BRIGHTNESS_SET_MIN_READ 25
Effect: Automatically saves to preferences

BRIGHTNESS_SET_MAX_READ <value>

Purpose: Set sensor reading threshold for normal brightness
Format: BRIGHTNESS_SET_MAX_READ <value>
Range: 0-65535
Example: BRIGHTNESS_SET_MAX_READ 100
Effect: Automatically saves to preferences

BRIGHTNESS_SET_MIN_PCT <value>

Purpose: Set display brightness percentage in dark room
Format: BRIGHTNESS_SET_MIN_PCT <value>
Range: 0-100
Example: BRIGHTNESS_SET_MIN_PCT 80
Effect: Automatically saves to preferences

BRIGHTNESS_SET_MAX_PCT <value>

Purpose: Set display brightness percentage in normal/bright light
Format: BRIGHTNESS_SET_MAX_PCT <value>
Range: 0-100
Example: BRIGHTNESS_SET_MAX_PCT 100
Effect: Automatically saves to preferences