IV-25 VFD Clock

Animation Interface Specification

# Purpose

This document contains everything needed to write animation code for the IV-25 VFD clock. All animation code goes in animation.ino. A fresh AI session can use this document alone to generate animations without needing the full system context.

# Platform and Environment

• Microcontroller: Arduino Nano ESP32

• Framework: Arduino (C++)

• RTOS: FreeRTOS (built into ESP32 Arduino core)

• Language: C++ with Arduino libraries

• Available functions:

- millis() - milliseconds since boot

- delay(ms) - blocking delay

- random(max) - random number 0 to max-1

- Standard C++ math: sin(), cos(), sqrt(), abs(), etc.

- memcpy(), memset() - memory operations

Code style: Standard Arduino C++ (.ino files are compiled as C++)

# CRITICAL: Variable Display Width

The display width is configurable! It depends on the number of boards:

• Width = numBoards * maxTube (e.g., 6 boards × 16 tubes = 96 columns)

• Height = Always 7 rows (fixed by IV-25 tube hardware)

• NEVER hardcode 96! Always calculate: int totalColumns = numBoards * maxTube;

# Hardware Overview

## Display Configuration

• Configurable width: numBoards * maxTube columns

- numBoards: Number of daisy-chained boards (default 6)

- maxTube: Tubes per board (always 16)

- Total columns: Typically 96 (6 boards × 16 tubes), but can vary

• Fixed height: 7 rows (all IV-25 tubes are 7 pixels tall)

• Physical layout: Multiple boards daisy-chained via RS485

- Board 0: Master (leftmost in physical space)

- Boards 1-N: Slaves

• Each board: 16 VFD tubes (IV-25)

• Each tube: 1 column × 7 pixels

## Important Variables

extern int numBoards;        // Number of boards (default 6)

extern const int maxTube;    // Tubes per board (always 16)

extern const int maxDot;     // Rows per tube (always 7)

// Calculate total width:

int totalColumns = numBoards * maxTube;  // e.g., 6 * 16 = 96

# The Display Buffer: allDisplayBits

## Declaration

extern bool allDisplayBits[96][7];  // Note: 96 assumes 6 boards

CRITICAL: The actual size is [numBoards * maxTube][maxDot]. Always calculate the width dynamically:

int totalColumns = numBoards * maxTube;  // Don't hardcode 96!

## Structure

• Type: 2D boolean array

• Dimensions: [totalColumns][7] where totalColumns = numBoards × 16

• Values:

- true = pixel ON (lit)

- false = pixel OFF (dark)

# Helper Functions

## Clear Display

void clearDisplay() {

int totalColumns = numBoards * maxTube;

for (int col = 0; col < totalColumns; col++)

for (int row = 0; row < 7; row++)

allDisplayBits[col][row] = false;

}

## Set Pixel (with bounds checking)

void setPixel(int col, int row, bool state) {

int totalColumns = numBoards * maxTube;

if (col >= 0 && col < totalColumns && row >= 0 && row < 7) {

allDisplayBits[col][row] = state;

}

}

# Animation Examples

## Rain Effect

void animateRain() {

int totalColumns = numBoards * maxTube;

// Scroll everything down one row

for (int row = 6; row > 0; row--) {

for (int col = 0; col < totalColumns; col++) {

allDisplayBits[col][row] = allDisplayBits[col][row - 1];

}

}

// Add new raindrops at top (random)

for (int col = 0; col < totalColumns; col++) {

allDisplayBits[col][0] = (random(100) < 10);  // 10% chance

}

}

## Wave Pattern

void animateWave() {

static float phase = 0;

int totalColumns = numBoards * maxTube;

for (int col = 0; col < totalColumns; col++) {

float wave = sin((col / 10.0) + phase) * 3 + 3.5;

int height = (int)wave;

for (int row = 0; row < 7; row++) {

allDisplayBits[col][row] = (row == height);

}

}

phase += 0.1;

}

# Summary

What you need to know:

1. Write to allDisplayBits[col][row] to change pixels

2. Check displayEnabled flag before animating

3. Display width is numBoards * maxTube columns (typically 96 for 6 boards)

4. Display height is always 7 rows

5. Column 0 = leftmost, Row 0 = top

6. Always calculate width dynamically: int totalColumns = numBoards * maxTube;

7. Create animation.ino with runAnimation() function

8. Add animation commands to console

9. Modify threadComputeDisplay to call your animation code

That's it! Everything else is handled by the existing system.