I have a bunch to discuss about filament power level.

First of all, we’ve discussed so far having the power-up sequence go from zero to maximum.  I would like to introduce a persistent filament brightness.  Instead of going to maximum, the duty cycle should go up to this persistent filament brightness (over 3 seconds during power-up).

Now we’re going to need mechanisms to set and maintain this persistent filament brightness (I’m going to call this PFB for now – please come up with a better name!).

To begin with the PBF should be stored in an ESP32 preference (please choose a good name for this preference) as a value between 1 and 100 (100 being a duty cycle of 100%).  If no such preference is found, it should default to 100.

Now about how to change this PBF.  We have to be able to do this from the Master board for all boards in the clock.  I propose that we add another command to the clock’s UI called “FILAMENT”.  This command will show the PBF for all the boards in the system, then prompt for a board number to change, then the new PBF value for that board (1 to 100).  The user should be able to iterate until aborting (I think a simple blank entry should be good enough for this).  What do you think of adding a possible value of “R” to the board number input which means reset the PFB on all boards to 100?

In order for this UI change to work, we’re going to have to add new RS485 commands.  I suggest adding the following commands:

RS485_READPFB: “$F” followed by "0" to "7" for the board number. This command requests the appropriate board to send back the PBF for that board (as read from the ESP32 preference on that board). Upon receiving this message, the appropriate board sends back its PFB. The format of the response is "$f" followed by a number between 1 and 100 as text.  (note the use of uppercase and lowercase “$F” and “$f” – I probably should have used this convention for the other RS485 message headers).

RS485_SETPFB: "$B" followed by "0" to "7" for the board number, followed by the PFB to write into that board.

Please make sure I have not caused a collision on the message headers.