Here is the next batch of changes I would like you to make:

- I would like a more robust startup approach across the master and all the slaves.  Here is my idea:  When a slave starts up (and it knows it is a slave from the DIP switch settings), it does not immediately startup the filament PWM.  That happens later.  The other threads start up, however.  RS485ReceiveCommand is called at some point to start processing messages.  However, if RS485ReceiveCommand sees a sequence of three “#” characters in a row on the RS485 bus, then RS485ReceiveCommand reboots the processor immediately.

The first time a RS485_RESPOND command is received, the slave will start up the filament PWM.

Now on the master, once it has started up, the first thing it does is send a stream of “#” characters into the RS485 bus.  I imagine a hundred or so characters should be sufficient and not too long.  It then goes into a several second wait before pinging all the slaves with the RS485_RESPOND command.

- There is a problem with startup:  If power is applied to both the slave and the master, the slave seems to come up fine because dots on its tube light up.  However, if the master is not plugged into my laptop, it appears to repeatedly crash and restart.  I can see the lights on the RS485 module blink on both the master and the slave, so this means the crash is happening after the master has identified the slave.  If I had to guess, this is the console thread on the master discovering that it can’t read its input.

- The clock hardware has both a light sensor (the TSL2591) and a BLANK pin on the MAX6921 chip.  I would you to create another background thread that will read the light sensor every second or so and adjust a PWM signal to the BLANK pin on the TSL2591.  Here is how this should work:

  - This background process should maintain a max read and min read value from the TSL2591.  These min and max values need a reasonable starting point (not too low for the min and not too high for the max).  Every time a new recent reading is taken from the TSL2591, the percent ratio of this reading to the delta between the max and the min is calculated.  So, if the new reading is close to the max, then this percentage should be close to 100%, and if the new reading is close to the min, then the percentage should be close to 0%.  This percentage should be further limited to no less than 10% and no more than 90%.  Once this percentage has been determined, it is time to configure the PWM for GPIO D8.  The frequency should be 10KHz, with a duty cycle determined by the calculated percentage.

- XXX