- I would like to improve the mechanism for adding animations.  Right now, this involves adding a file called “animations.ino” and making a code change to threadComputeDisplay.  I should not have to make a code change.  I suggest that you make a file called “animations.ino” that is loaded with your default clock display code.  Then, when I create a new animations.ino file (with your help, of course, and in a different session), all I have to do is replace this animations.ino file with the new one.

- I am concerned about the transmission time for sending tube updates.  Animations like scrolling will likely require updating all the tubes in the system.  With 7 slaves, each with 16 tubes, that’s a lot of data to transmit.  So I would like to do 2 things.  The easy thing is to increase the baud rate substantially, at least a factor of 8, maybe 16.  Second is to introduce a new command that includes all the data for all the tubes on a board.  When SendDisplay determines that a tube has changed, instead of just sending that tube, SendDisplay should send data for all the tubes on that board.  And furthermore, the data for a single tube in that packet should be changed from one byte per dot to 2 hex characters for the tube.  So this new command would have 16 * (2 * 7) bytes of data.  The command would include the board number and that data, so the total length would be 2 + 16*(2*7) + 1.

- I would also like to restructure the code for the proximity detection.  I would like to use this code in other projects that also use the VL53L1X.  So I would like you to figure out everything that is needed to make this work and to put it into a file called proximity.ino.  This file should have sufficient comments so that you, Claude, can read it in a new session and integrate it into a totally new or different program.  This proximity code will have to make some assumptions about this integration.  A key thing to figure out is whether calls to the VL53L1X sensor belong in this code.  Or is this code going to get called with the data returned by the sensor.  Please think hard about the best way to design the interface for this code.